## PU-ES30

Technical data – bidder details

# TOILET CUBICLE SYSTEME

| MODEL: TYPE PU-ES30 of SCHÄFER Trennwandsysteme GmbH, D-56593 Horhausen, Phone (+49) 2687/915110 or technical and visual absolutely equal. DESIGN: The partition panels, uprights, pilasters and doors are made in sandwich construction. All corners and edges are rounded for safety. The sandwich panels are strengthened with an internal all-round frame made of rigid PVC sections. The cavity between the panels is completely filled with CFC-free polyurethane foam which permanently bonds the two skins together keeping them flat and parallel. The panels are trimmed with profiles. MATERIAL: stainless steel DIN 1.4301, structured, non-coated, rust-resistant, surface D21. DOORS: The doors are kept closed by concealed, solid plastic, spring-loaded hinges integrated in the top rail section. |  | TOP RAIL SECTION: Aluminium profile with plastic cover. WALL MOUNTING AND CORNER CONNECTION: The element is fixed to the wall using an aluminium channel. The corner connection links the front wall to a side wall and is made from an extruded aluminium section. HARDWARE: Fully galvanized lock with latch and bar. Safety handle u-shape made of unbreakable nylon with steel core, 23 mm thick. Rosettes, vacant/engaged indicator and an emergency unlocking device on the outside. FEET: The fully adjustable legs with welded-on base and flat cover plate are completely made of stainless steel DIN 1.4301. STANDARD ACCESSORIES: One coat hook and one door buffer per cubicle. COLOUR: According to manufactures´ colour chart HEIGHT: Standard height 2000 mm incl. 150 mm floor clearance |
| --- | --- | --- |