PUP40 Altus

Technical data – bidder details

TOILET CUBICLE SYSTEM

| MODEL: TYPE PUP40 Altus of SCHÄFER Trennwandsysteme GmbH, D-56593 Horhausen, Phone (+49) 2687/915110 or technical and visual absolutely equal. DESIGN: Room-high toilet partitions. The partition panels, uprights, pilasters and doors are made in sandwich construction. The cavity between the panels is completely filled with CFC-free polyurethane foam which permanently bonds the two skins together keeping them flat and parallel. The panels are welted on all sides so that no profiles need to be applied and the front is completely flush-fitting. Total thickness 40 mm. MATERIAL: Steel, hot-dip galvanised, powder coated, designed for rooms with high moisture content. The surface of the panels is very resistant to scratches and hits. Furthermore, it is colour-proof and resistant to heat. DOORS: The doors are welted and without applied profiles. HEADRAIL SECTION: Aluminium profile powder coated in the same colour of the panels. It runs along the front and the side walls of the cubicle. |  | WALL MOUNTING AND CORNER CONNECTION: The element is fixed to the wall using an aluminium channel. Alternatively, it can be executed with a shadow gap. The corner connection links the front wall to a side wall and is made from an extruded aluminium section. HARDWARE: Two stable stainless steel hinges per door. Also available in self-closing function. Hinge lugs are fitted flush into the panels. Fully galvanised lock with latch and bolt. An U-shaped safety handle of unbreakable nylon with rosettes, an olive latch and, on the outside, a vacant/engaged indicator and an emergency unlocking device. STANDARD ACCESSORIES: One coat hook and one door buffer per cubicle. COLOURS: According to manufactures´ colour chart HEIGHT: Room-high cubicles, maximum height 2.700mm |
| --- | --- | --- |