## VITRUM GS

## Technical data – bidder details

LOCKER

PRODUCT:

TYPE VITRUM GS from SCHÄFER Trennwandsysteme GmbH, D-56593 Horhausen, Tel. +49 2687 91510 or technically and optically exact equivalent. The system is certificated by the German Association for Technical Inspection and has the GS symbol. Systems without this symbol are not admitted.

To prove the sustainability of the product, the system must be PEFC™ or FSC™ certified. The corresponding certificate must be presented. Systems without valid PEFC™ or FSC™ certification are not allowed.

DESIGN:

Moisture resistant wardrobe made of solid grade laminate panels without protruding aluminium profile, with flush-fitting appearance and doors with ceramic screen-printing, scratch-proof safety glass (Float) with necessary Heat-Soak-test (ESG-H).

LOCKER BODIES:

Solid grade laminate panels absolutely moisture resistant, rot-proof, scratch and impact resistant sturdily fitted together. Back and dividing wall 4 mm thick, top and bottom 8 mm thick. Connection of back wall to side wall through rounded aluminium profiles, white powder-coated. Aluminium instead of solid grade laminate is not permitted. The construction must make it possible, by means of variable shelf width, to adapt the number of lockers to requirements at construction sites. Ventilation by set-back floor and roof. Decor side walls 10 mm thick, safety glass. The use of silicon inside the locker is not permitted.

DOORS:

Doors of 10 mm thickness, ceramic screen-printing, scratch-proof safety glass (float) with heat-soak test (ESG-H). All visible edges polished flat.

2 stable hinges from anodised aluminium with stainless steel pivots and bands of high-performance polymer . Opening limit of the doors to an angle of 95° by fitting aluminium sections into the front. Doors without an opening limit are not permitted. On the side with hinges. The form of the wardrobe sections must guarantee that upon opening the door no fingers can be pinched.

STANDARD DOOR CLOSING:

Security cylinder lever lock as master key system.

Mounting of lock on aluminium base panel which simultaneously serves as an anti-theft measure after closing the door by fitting it into the side sections. Aluminium outer projecting round rosettes with integrated flush-fitting number-plates with engraved numbers.

INTERIOR FITTINGS:

- One-piece locker with hat rack made of from 13 mm thick solid grade laminate panels, underneath a continuous aluminium cylindrical pipe hanging rail with two-way sliding hooks. Doors equipped with an anti-theft system

- multipart lockers with shelf made of 13mm solid core panels each compartment with triple hook

- deposit boxes with shelves made of 13 mm solid grade laminate without any interior.

COLOURS:

Locker body is white. Doors and decorative end panels as well as aluminium round profiles are according to manufacturer’s colour chart.

STANDARD DIMENSIONS:

Width:	300 mm

Depth:	500 mm

Height:	1850 mm	without subframe

INSTALLATION:

- installation on a plinth existing on site

- Installation of lockers on a plinth made of

aluminium tubes, d=40 mm, powder-coated, with adjustable screw feet, 150 mm high.

- Installaiton of lockers on a construction est. 430 mm high with aluminium tubes D=40mm incl. bench seating.

MATERIALS

All fittings must be corrosion-proof, anodised (E6/EV1) and 100% recyclable. Plate finished aluminium parts are not permitted. All glass colourings to be lead- and cadmium-free, 100% recyclable.