## VITRUM II WK

## Technical data – bidder details

CHANGING CUBICLE SYSTEM

PRODUCT

TYPE VITRUM II WK of SCHÄFER Trenn

wandsysteme GmbH, 56593 Horhausen, Tel.: 00+49 2687 9151-0, or technically and optically exact equivalent. The system is certificated by the German Association for Technical Inspection and has the GS symbol. Systems without this symbol are not admitted.

DESIGN:

Enamelled, scratch-proof SCHÄFER safety glass with ceramic printing and necessary heat soak test (ESG-H) fitted to natural anodized aluminium connecting elements and fittings.

WALLS AND DOORS:

Enamelled SCHÄFER safety glass (float), 10 mm thick, resistant to stains. All corners polished smooth. The entire installation is

resistant to corrosion and sea water.

CONSTRUCTION:

Free-hanging cubicle unit installed on a cubicle central foot. The foot is made of a hot-dip galvanized steel which will be installed on the non-fines concrete with four screws and anchor bolts.

During the final installation the steel foot will be covered up with an aluminium round pipe.

Sealing between floor and pipe end with neoprene rubber sealing.

On the hot-dip galvanized steel foot the steel bench seating construction will be installed as well. All glass elements will be installed on this construction.

An aluminium round profile (d=30mm) runs

above the front wall, wall thickness 5mm,

connection with aluminium clamps which hold and stabile the glass elements.

Wall connection with aluminium clamps. The bottom clamp is closed to avoid that the glass will fall down.

All element connections are glued and additionally screwed as well. All screw connections are made with safety TORX screws made of stainless steel and with an safety pin.

Other screws are not permitted.

HARDWARE:

Self-closing doors using two maintenance-free aluminium stirrup hinges with stainless steel pivots and hinges of high-strength polymer. Hinges with springs are not permitted. Doors alternatively self-closing or self-opening.

Hinges glued on to the glass surfaces and in addition fastened with screws. Protection against pinched fingers and visual cover provided by aluminium cylindrical pipes on the hinge side.

LOCKING/SEATING

Central locking underneath the bench seating. Unlocking with stainless steel rocker arm next to the bench. Engaged/vacancy indicator and emergency unlocking device on the outside of the cubicles. This ensures that in case of an emergency an employee can open the door as well. Central locking without an emergency unlocking device is not permitted.

Bench seating made of 13mm thick solid grade laminate, absolutely moisture resistant, rot proof, scratch and impact resistant. All edges rounded.

STANDARD ACCESSORIES:

Two hooks, one mirror 400 x 300mm and one shelf made of 26mm thick solid grade laminate incl. solid rubber as a door buffer

COLOURS

Glass elements according to manufacturers‘ colour-chart. All fittings natural anodized E6/EV1.

HEIGHT

Headroom 2010 mm incl.250 mm clearance.

MATERIALS

All fittings must be corrosion-proof, anodised (E6/EV1) and 100% recyclable. Plate finished aluminium parts are not permitted. All glass colourings to be lead- and cadmium-free, 100% recyclable.