VITRUM II

# Technical Data – bidder details

TOILET CUBICLE SYSTEM

PRODUCT:

TYPE VITRUM II from  SCHÄFER Trennwandsysteme GmbH, 56593 Horhausen, Tel.: (+49) 2687 / 91510, or technically and optically exact equivalent. The system is certificated by the German Association for Technical Inspection and has the GS symbol. Systems without this symbol are not admitted.

DESIGN:

Ceramic screen-printing, scratch-proof safety glass fitted to anodised aluminium connecting elements and fittings.

WALLS AND DOORS:

Ceramic screen-printing safety glass, (ESG-H) 10 mm thick, resistant to stains and with the necessary heat-soak test. All corners polished smooth. Entire installation to be resistant to corrosion and sea water.

CONSTRUCTION:

A horizontal continuous aluminum profile 40/20/2, running along the front is recessed by 120 mm, with aluminium where the front elements are fixed and stabilized. Per dividing wall 4 massive angles to connect the front wall with the dividing wall. Feet are recessed, too, so that the cubicles seem to be floating. Wall connection of the front elements by 3 massive aluminium angles which are fixed hidden on the inside of the cubicle. Other wall connections by aluminium clambs, bottom clamb always closed to avoid a skidding down of the glass panels. All screw attachments to be stainless steel safety TORX screws with a security pin put in the screw opening. Other screws are not permitted.

HARDWARE:

Self-closing doors using two maintenance-free aluminium rising hinges made of high-performance polymere and with stainless steel pivots. Spring hinges are not permitted. Doors alternatively self-closing or self-opening. Protection against pinched fingers and visual cover provided by aluminium cylindrical pipes on the hinge side. Visual cover on the lock side provided by glued-on aluminium ridges which are additionally held by the lock and the door catch. Ridges

which are only glued-on are not permitted. Bolt lock of anodised aluminium with stainless steel bolt. Locks with plastic bolts are not permitted. Locking by means of SCHÄFER aluminium one-hand fittings. Inside easily graspable locking wheel, outside aluminium button with integrated engaged/vacancy indicator and emergency unlocking device. Locks without emergency unlocking devices are not permitted.

FEET:

Feet recessed to the front side by 120 mm. On the floor, screwed-in anodised aluminium panels with welded-on cylindrical bars (solid), Covering with anodised aluminium rosette. Anodised aluminium foot-clamp with insert nut, in which the glass elements are to be framed.

STANDARD ACCESSORIES

One hook, one door buffer per cubicle.

COLOURS:

Glass elements according to manufacturers’ colour-chart. All fittings natural colour and anodised E6/EV1.

HEIGHT:

Headroom 2012 mm incl. 100 mm clearance. Glass ends at 2000 mm, top edge of wall fixing at 2045 mm.

MATERIALS:

All fittings must be corrosion-proof, anodised (E6/EV1) and 100% recyclable. Plate finished aluminium parts are not permitted. All glass colourings to be lead- and cadmium-free, 100% recyclable.